## Frequently Asked Questions for Renovators and Homeowners

What is asbestos?
Asbestos is the generic term for a number of fibrous silicate minerals. Products made from asbestos cement - bonded asbestos material - include fibro sheeting (flat and corrugated) as well as water, drainage and flue pipes, roofing shingles and guttering.

Only fibro products made before 1987 contain asbestos. The manufacture and use of asbestos products was banned nationally from 31 December 2003.

How can asbestos affect my health?
Breathing in asbestos fibres can cause asbestosis, lung cancer and mesothelioma. The risk of contracting these diseases increases with the number of fibres inhaled and the risk of lung cancer from inhaling asbestos fibres is also greater if you smoke. People who get health problems from inhaling asbestos have usually been exposed to high levels of asbestos for a long time. The symptoms of these diseases do not usually appear until about 20 to 30 years after the first exposure to asbestos.

What are the health risks for renovators?
Most people are exposed to very small amounts of asbestos as they go about their daily lives and do not develop asbestos-related health problems. Finding that your home or workplace is made from fibro products does not mean your health is at risk. Studies have shown that these products, if in sound condition and left undisturbed, are not a significant health risk. If the asbestos fibres remain firmly bound in cement, generally you do not need to remove the fibro or even coat it.

Health problems can occur when people are unaware of the hazards of working with fibro and do not take appropriate precautions. The important point is to always work to avoid or minimise the release of dust or small particles from asbestos material. If you use commonsense and follow basic safety guidelines, working with fibro products should not be a problem.

How do I know if my fibro contains asbestos?
The most accurate way to find out if your fibro contains asbestos is to have a licensed asbestos removal contractor inspect and test it. You can't tell by looking at it. If in doubt, assume asbestos is present and take the necessary precautions..

I have concerns about the neighbours (or a contractor working for them) taking down a shed or demolishing a house and the dust generated. Are they doing it safely?
Your neighbour, or their contractor, should be:

- wearing personal protective equipment

- taking the sheets off whole and not using power tools to minimise dust

- not working on windy days

- wetting down the sheets

- putting them in a plastic-lined skip.