HAND SWAGER OPERATION INSTRUCTIONS Figure 1 Figure 1. Place sleeve to be compressed in proper size groove in swaging tool. Single cavity tools should match cable size to be swaged. Length of cable equal to the cable diameter should extend beyond length of sleeve to achieve maximum holding. (See arrow in figure 3) Figure 2 Figure 2. Keep jaws of swaging tool at right angles to the sleeve to be compressed, making sure the sleeve is aligned in the jaw grooves. Close tool completely. Tool handles should snap shut indicating complete closure. Figure 3 Figure 3. Finished sleeve should look like this. Be sure sleeves have been compressed the proper number of times. (See NUMBER OF COMPRESSIONS table below) Figure 4 Figure 4. Use Swaging gauge provided with the tool to check proper after-swage diameter. Compressed sleeve should slide freely into size slot of gauge.

| NUMBER OF COMPRESSIONS |  |  |  |  |
| --- | --- | --- | --- | --- |
| CABLE DIAMETER | ALUMINUM & COPPER |  |  | *STAINLESS STEEL |
|  | OVAL SLEEVES | STOP SLEEVES |  | OVAL SLEEVES |
| 3/64" | 2 | 1 |  | 1 |
| 1/16" | 2 | 1 |  | 1 |
| 3/32" | 2 | 2 |  | 1 |
| 1/8" | 3 | 2 |  | 2 |
| 5/32" | 3 | 2 |  | 3 |
| 3/16" | 4 | 2 |  | 4 |
| 7/32" | 4 | 2 |  | 4 |
| 1/4" | 4 | 3 |  | 5 |
| 9/32" | 4 | 3 |  | N/A |
| 5/16" | 5A / 4C | 3 |  | N/A |
| 3/8" | N/A | 3 |  | N/A |